Rubberecycle LLC.

1985 Rutgers University Blvd.

Lakewood, NJ  08701

USA

Toll Free	(888) 436-6846

Phone	(732) 363-0600

Fax	(732) 370-4247

Website	www.rubberecycle.com

E-mail	info@rubberecycle.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 1995 Edition, with numbers and titles from MasterFormat 2004 Edition in parentheses.  Delete version not required.

SECTION 02791 (32 18 16.13)

PLAYGROUND SURFACING SYSTEM

Specifier Notes:  This section covers Rubberecycles “Accessamat” playground surfacing system and accessible walkway tile. Consult Rubberecycle for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Rubber, interlocking playground and walkway tile.

1.2	RELATED SECTIONS

Specifier Notes:  Edit the following list of related sections as required for the project.  List other sections with work directly related to this section.

Include the section number and title of the following two Division 2 (Division 32) sections, if they are referenced in this section.

A.	Section 02792 (32 18 16.13) – Poured-in-Place Playground Surfacing.

B.	Section 02880 (11 68 13) – Playground Equipment.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

- ASTM F 1951: Standard Specification for Determination of Accessibility od Surface Systems Under and Around Playground Equipment.

B.	ASTM F 1292 – Standard Specification for Impact Attenuation of Surface Systems Under and Around Playground Equipment.

C.	US Consumer Product Safety Commission (CPSC) Handbook for Playground Safety.

1.4	SUBMITTALS

A.	Comply with Section 01330 (01 33 00) – Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, including installation and subsurface instructions.

C.	Samples:  Submit manufacturer's sample of at least 6”X6” in size.

D.	Test Reports:  Submit certified test reports from qualified independent testing agency indicating results of the following tests:

1.	Impact Attenuation:  ASTM F 1292.

2.	Accessibility:  ASTM F 1951

E.	Certificate of Compliance:  Submit manufacturer's certificate of compliance indicating materials comply with specified requirements.

F.	Manufacturer's Project References:

1.	Submit list of successfully completed projects.

2.	Include project name and location, name of architect, and type and quantity of playground safety surfacing furnished.

G.	Maintenance Instructions:  Submit manufacturer's maintenance instructions.

H.	Warranty:  Submit manufacturer's standard warranty.

1.5	QUALITY ASSURANCE

A.	Installer's Qualifications:

1.	Approved by manufacturer.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

- Storage:

1.	Store materials in accordance with manufacturer's instructions.

C.	Handling:  Protect materials during handling and installation to prevent damage.

1.7	WARRANTY

A.	Materials and Workmanship:  Playground surfacing and accessible walkway pads shall be warranted for defects in materials and workmanship for 5 years from date of completed installation.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Rubberecycle, LLC., 1985 Rutgers University Blvd.  Lakewood, New Jersey, 08701, USA.  Toll Free (888) 436-6846.  Phone (732) 363-0600.  Fax (732) 370-4247.  Website www.rubberecycle.com.  E-mail info@rubbermulch.com.

2.2	PLAYGROUND SURFACING SYSTEM

A.	Tiles:  “Accessamat”

1.	Description:  Compression Molded, interlocking, tile system

3.	Material:  SBR rubber.

4.	Tile Locking:  Shiplap perimeter configuration with predrilled holes to rivet tiles together

5.	Configuration:  Hollow cavity reverse pyramid tray consisting of 64 cavities

6.      Size:  36””X 60”” Nominal.

7.	Thickness: 2.0”

8.      Weight per tile: 40 lbs.

B.	Test Results:

- Impact Attenuation, ASTM F1292:

- Gmax score less than 200

- HIC score less than 1000

2.     Accessibility, ASTM F 1951:  Pass

PART 3     EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive playground surfacing and accessible walkway tiles.  Notify Architect if areas are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.2	PREPARATION

A.	Prepare subsurface in accordance with manufacturer's instructions to ensure proper slope, support and drainage for playground surfacing system.

3.3	INSTALLATION

A.	Install tile system in accordance with manufacturer's instructions at locations indicated on the Drawings.

B.	Ensure prepared subsurface and tiles are dry and clean.

C.     Layout tile surface in accordance with manufacturer's instructions.

D.	Install rivets in accordance with manufacturer's instructions.

3.5	PROTECTION

A.	Protect completed surface from damage during construction.

END OF SECTION